Product Name: DMN104; fischer FIS EM Plus 390 S Injection mortar Epoxy 390ml 550g

Lower price, replacement warranty.

Price Tk.4200 (price negotiable)

Product Details:

The item is a Fischer FIS EM Plus 390 S injection mortar cartridge.

- This is a high-performance epoxy resin mortar designed for maximum load capacity in cracked and non-cracked concrete.

- It is suitable for securing rebar connections and heavy-duty anchoring applications.

- The product has a 390 ml capacity and is approved for use in diamond-drilled or water-filled drill holes.

- It can be applied in cold temperatures, down to -5°C.

Product Features

- FIS EM Plus with optimized formulation for improved load values in cracked and non-cracked concrete.

- The ETA assessment guarantees a service life of 100 years, an external expert report even up to 120 years.

- For rebar connections from diameter 8 to 40 mm.

- Can be used with threaded rod FIS A or internal threaded anchor RG M I for temporary and removable fixing points.

- FIS EM Plus is to be processed at temperatures down to -5°C.

- More safety due to approvals for diamond drilled and water-filled drill holes as well as seismic applications of performance categories C1, C2.

- The approval of the injection mortar for use in glued laminated or glued solid timber of spruce (Picea abies), fir (Abies alba) or pine (Pinus sylvestris) allows the realisation of high-performance and appealing wood-wood, wood-concrete and wood-steel connections.

Applications

- Post-installed rebar connections

- Jointers for concrete layers

- Rim beam anchorings

- Seismic applications

- Anchorings in diamond-drilled or in water filled drill holes

- Heavy steel constructions

- Silo installations

- Tall shelving

- Sound barriers

- Temporary or removable fixings (with internal threaded anchor RG M I)

- Anchorings in sealing surfaces according to WRL (Water Resources Law)

- Glued anchorages in wood, see approval for gluing in wood